Atheros WiFi Authentication Software – Version apps/wpa2

Features and Configuration

Last Updated: 2008-08-12 (Ted Merrill)

TODO: put this into Atheros documentation template.

Scope and Audience

This document explains the supported features of Atheros apps/wpa2 software and how to configure the software to obtain the features.  It is intended for use by Atheros customers and within Atheros.

Introduction

The apps/wpa2 software provides the WiFi user space authentication component for a Linux system using Atheros WiFi hardware.

To put this into context better, consider that control of WiFi (IEEE standard 802.11) can be conceptually divided into four layers:

- Hardware – (only Atheros WiFi hardware is supported.)

- Kernel driver – provides strongly real time control of the hardware. Such drivers must provide certain features expected by apps/wpa2.

- User space authentication (here, provided by apps/wpa2)– provides services that need run only sporadically and without strong real time requirement, mostly but not entirely related to authentication.  This for convenience includes feeding configuration to the kernel.

- User interface – e.g. for setting up configuration and triggering configuration changes (must be compatible with the interface expected by apps/wpa2).

The Atheros apps/wpa2 software is based upon version 0.5.9 (may be upgraded to 0.5.10) of the free software pair hostapd and wpa_supplicant (refer to http://hostap.epitest.fi) as modified by Sony and further modified by Atheros.  This is intended to replace previous apps/wpa software based upon earlier hostapd versions with add-on programs for WPS (WiFi Protected Setup) support.  Compared with apps/wpa, the apps/wpa2 software has the advantage of more up-to-date features, while having WPS integrally supported within the hostap software resulting in smaller resource consumption and easier maintenance.  The previously used add on programs (wsccmd and wps_enrollee) are no longer used.  The apps/wpa2 software consists of two primary programs:

- hostapd provides access point initialization and authentication of WiFi stations.

- wpa_supplicant provides WiFi station support for authentication.

In addition to these two primary programs, there are a number of small utility programs:

- Various “wireless_tools” programs which are provided for startup shell scripts and for debugging.  With a little work, the functionality could be pulled into a bigger program and the shell scripts eliminated.

- wpatalk allows sending commands to hostapd and wpa_supplicant.  In a real product, a web server program should send said commands.

Features NOT supported

The following access point features have some source code support but are specifically not supported by Atheros at this time. Customers attempting to make use of these features should not expect support from Atheros.

- Hardware other than Atheros hardware is not supported by Atheros.

- Simultaneous support of more than one authentication mode (open, WEP, WPA/PSK, WPA-enterprise variants) on the same virtual AP is not supported.  (Except that WPA and WPA2 authentication and TKIP and CCMP encryption ARE simultaneously supported).

- Unique pre-shared keys per station.  This feature is provided for in the source code, but more thought needs to be given as to how this would be supported.  Note that this would only affect the pairwise keying and not group (i.e. broadcast) keying. Adding this feature could greatly improve the security especially when using mixed tkip and ccmp encryption is used (where tkip provides a security hole by which the pre-shared key can be learned) and in general where the pre-shared key might be compromised.

- Station mac (address) based authentication using accept and deny lists of addresses. Comments indicate that this won't work as written for madwifi (i.e. Atheros) drivers.

- External EAP / radius server is supported only with WPS disabled.

- IAPP not supported.

- Serving by the internal WPS registrar of configuration parameters via WPS other than those currently in use on the access point, is not supported.  This does not preclude using an external upnp-connected WPS registrar for such purposes (e.g. connecting a supplicant to a different SSID depending on the type of device the supplicant is).

- WPS using NFC (near field control) or USB (universal serial bus) is not supported.  NFC code in the source code has never been tested by Atheros and has probably seen significant bit rot.

- IBSS (aka peer to peer) configurations are not supported.

- WPS via upnp is supported only for hostapd, not for wpa_supplicant (thus wpa_supplicant cannot serve as an external registrar.  This could be fixed by writing a “tiny” upnp/wps support code for wpa_supplicant, but there is no apparent application.

- wpa_supplicant has code support for letting the driver do selection of access point, but this is NOT supported by Atheros.  The wpa_supplicant config parameter ap_scan should remain at the default value of 1 (e.g. by omitting it from the configuration file).

- WPS must be disabled (wps_disable=1) if enterprise authentication is enabled

Features that may be Disabled at Build Time

The following features may be disabled by the customer at build time to save image size etc.:

- WPS

- UPnP use with WPS

- Enterprise authentication methods

Feature Overview

A question mark before a feature means that it is potentially supported, but we haven't decided whether it will be officially supported by Atheros.

IMPORTANT: the presence of feature support in software source code does not mean that it is supported by Atheros.

General features:

- The topology configuration file defines which real hardware and virtual access points are to be used for what.  See apps/examples/topology.conf  and below.  This file must be passed as the basic input to hostapd and wpa_supplicant programs, and to an optional startup program “apstart”.  Optimal software code for startup purposes is still TODO.

Topology file format:

- Bridge.  Per Linux traditions, bridges are named br0, br1 etc. and are network interfaces in the Linux kernel sense.  (A bridge named “none” can also be used as a place holder for unbridged virtual access points, if any).  Each bridge has an IP address and a list of network interfaces that are connected by the bridge.  (Although any network interface can be assigned an IP address, when bridges are used only the IP address of the bridges have meaning).  “Virtual access point” and wired network interfaces are listed per bridge.  Virtual access point network interfaces based on Atheros hardware are named ath0, ath1 etc. and can (in spite of the name) provide a WiFi station instead of an access point.  Wired ethernet network interfaces are typically named eth0, eth1 etc.  All network interfaces listed for a bridge have their traffic joined together by the bridge, and effectively share the IP address of the bridge.

- Radio.  Per Atheros driver conventions, WiFi radios are represented by Linux network interfaces named wifi0, wifi1, etc. Each radio supports a number of virtual access points which, in spite of the name, can actually be WiFi stations.  Beware of a number of builtin limitations to how virtual access points can be used (which is beyond the scope of this document). Subsections of radio:

  - Virtual access points which are actually access points (vs. stations) and which share the same basic radio parameters should be contained in the same “ap” section along with the name of the access point radio configuration file (see ap_example_80211g.conf for an example).  Each such virtual access point is listed within the “ap” section as a “bss” identified by the interface name of the virtual AP (ath0, ath1 etc.); a per-BSS configuration file is associated with each (can be shared by more than one bss).  TODO: would it ever make sense to have more than one “ap” section per “radio”?

  - Virtual access points which are actually stations are listed with “sta” entries with the interface name (ath0 etc.) and station configuration file (which can be shared by multiple stations).

Per-radio access point features:

NOTE: most or all per-radio features have been disabled from hostapd.  It is intended that such features be enabled by other software prior to running hostapd, and then left unchanged.  In some cases (which?) it is necessary for hostapd to know the setting used,  in which case the setting must be included in the per-radio configuration file.  TODO: what is the exact list?

- ? frequency band and communication standard: ieee80211.b or 11.g for 2.4GHz band, ieee80211.a for 5 GHz band.

- ? Selection of regulatory domain and restriction of channels used accordingly.

- ? ieee802.11d feature: advertisement of regulatory domain / country code (so stations can modify their behavior to match regulatory regulations).

- ? Channel selection: specific channel, or automatic selection.

- ? Beacon interval (default 100 msec)

- ? RTS/CTS threshold.

- ? Packet fragmentation threshold.

- ? WMM (wireless multi-media) parameters.

- ? Internal bridging control (bridging between wireless devices done at the wireless kernel level).

Per-BSS access point features:

Please refer to examples/config1.ap_bss for explicit instructions on setting of individual fields!

- Multiple BSS per radio, implemented by (and limited by) the kernel driver virtual access point implementation, thus a fully symmetric implementation.

- The unique bssid (aka mac address) of the BSS is defined or invented by the hardware or kernel code and is not part of the apps/wpa2 configuration.

- A per-BSS configuration (file) can be shared by and used for multiple BSS on same platform, which is particularly useful for providing simultaneous compatible BSSs on different radio bands.

- SSID (service set identifier) per ieee802.11

- DTIM interval (in beacons)

- ? Hidden SSID.

- ? WMM (wireless multi-media) support.

- Station inactivity timeout.

- Choice of major authentication modes with encryption submodes:

  - Open/unencrypted

  - Open/WEP encryption

  - WEP shared key authentication with WEP encryption

  - WPA and/or WPA2 PSK with TKIP and/or CCMP (aka AES) encryption.

  - WEP-8021x {EAP TLS / EAP TTLS / PEAP MSCHAPv2 / PEAP GTC / EAP FAST}

  - WPA-8021x {EAP TLS / EAP TTLS / PEAP MSCHAPv2 / PEAP GTC / EAP FAST}

  - WPA2-8021x {EAP TLS / EAP TTLS / PEAP MSCHAPv2 / PEAP GTC / EAP FAST}

- WEP

  - encryption: four keys plus default key index per ieee802.11.

  - WEP rekeying control.

- WPA[2]

  - pre-shared key and other parameters.

  - appropriate information elements in beacons and probe responses

  - hostapd handles authentication message interchange with station

- ? ieee802.11w parameter?

- WPS (Wi-Fi Protected Setup)

  - See apps/wpa2/examples/config1.conf  for field descriptions.

  - Configuration field to disable use of WPS entirely.

  - Configuration field to disable use of UPnP with WPS.

  - The access point configuration may be changed by WPS operation if the peer so desires, if and only if one of the following applies (note the possible security hole while wps_configured is zero; however the PIN plus locking on repeated failures should protect):

    - The wps_configured field is set as zero.

    - or, The CONFIGME command was used.

  - The wps_configured field is always set to one if the configuration is rewritten as result of WPS operation.

  - Supported authentication/encryption methods that can be shared via WPS:

    - Open / unencrypted

    - Open / WEP encrypted

    - ? WEP encryption plus enterprise methods ? TODO expand this

    - WPA or WPA2 authentication with TKIP and/or CCMP encryption and PSK or TODO ? enterprise methods requiring further user input?

  - Only the authentication/encryption methods currently in use on the access point BSS (and which can be shared via WPS) will be shared.

  - WPS session types:

    - WPS EAP session: begins with request from station, and is ended on access point on when one of various errors are encountered, or when the station sends a negative acknowledgement (NAK), or when WPS is deemed successful.  There may be only one EAP session per station.   ? Timeout on a WPS EAP session.

    - WPS meta session (aka WPS job): begins with activation and continues until a WPS EAP session succeeds (see above), the meta session is cancelled via command to hostapd, or a timeout is reached.  Meta sessions are activated by a CONFIGTHEM or CONFIGME command to hostapd, or receipt of a Selected Registrar message via UpnP from an external registrar. The meta-session status is advertised in beacons and probe requests via the Selected Registrar information element.  Failed WPS EAP sessions are ignored during the meta-session.  When a WPS meta-session is in progress, any requests by a station for a WPS EAP session are considered in terms of  the meta-session; the meta-session may be aborted if there appears to be a conflict in requests or there would be more than one ongoing EAP session.  Meta-sessions are aborted on receipt of the CONFIGSTOP command or on receipt of an incompatible CONFIGTHEM or CONFIGME command, or a Registrar Unselect Upnp message from external registrar. Compatible CONFIGTHEM or CONFIGME commands have all the same major modes and same PIN, and result in extension of the timeout instead of  session termination.  Meta-sessions are also aborted on timeout.  The timeout defaults to 2 minutes, but a different timeout may be specified with the CONFIGTHEM or CONFIGME command. WPS EAP sessions in progress at termination of a meta-session are aborted. A meta-session started with the CONFIGTHEM command is now allowed to reconfigure the access point.  (Due to the way the WPS protocol is defined, a meta-session started with CONFIGME could actually be used by the station to configure itself and not configure the access point). Per the WPS standard, if the access point is configured, the wps_configured field will be set in the hostapd per-bss configuration file.  In addition, when the access point BSS is configured, the entire hostapd process is automatically restarted.

  - Methods of activation:

    - Local push-button method: on command (CONFIGTHEM) to hostapd, a WPS meta-session is provided wherein a station can enlist WPS support from the access point with PIN=00000000.  This push button method is advertised in beacons and probe responses. Note that according to WPS standard, the timeout should be 2 minutes; use of a different timeout is optional and non-standard.  In conformance with the WPS specification, CONFIGME is not allowed with push button method.

    - Local PIN entry method.: on command (CONFIGTHEM or CONFIGME) to hostapd, a WPS meta-session is provided wherein a station can enlist WPS support from the access point with a PIN (password) that must be entered by user on both access point and station.  The WPS specification does not specify a timeout, but having one is a good idea.

    - Any other host on the network with IP connectivity announces via UPnP that it is the selected registar (a proxied session is established).

    - “Label method”:  WPS requests from a station that are received when not in a meta-session as described above can be enabled in the configuration file to use a “default PIN”.  No meta-session is created, and any number of EAP sessions may be simultaneously active (but no more than one per station).  The support consists only of providing parameters to a station unless hostapd is configured as “wps_configured=0” in which case it also will receive new parameters from an external registar station and write them to the per-BSS configuration file (and hostapd will restart itself).

  - WPS medium:

    - WPS over WiFi per WPS specification. May be independently enabled/disabled.

    - WPS over UpnP, which is internet protocol (IP) based, and will be carried by the operating system over whatever link (wired or wireless) is operational and selected as the route to use by the operating system.  May be independently enabled/disabled.

General supplicant (station) features:

- The configuration file can be locked by setting update_config=0.  Any changes to configuration are not saved and are lost on restart of wpa_supplicant. This has an obvious conflict with WPS.

Supplicant (station) wps configuration:

Please refer to comments in apps/wpa2/examples/config1.sta for description of WPS parameters and refer to apps/wpa2/wpa_supplicant/wpa_supplicant.conf for a description of other parameters):

- Optional uuid.  It is reccommended to omit this and to let wpa_supplicant invent one based on the mac address of the station.

- auth_type_flags: per WPS spec, a bit set of possible authentication methods. The access point in a WPS operation could honor this if there were choices.

- encr_type_flags: per WPS spec, a bit set of possible encryption methods.  The access point in a WPS operation could honor this if there were choices.

- conn_type_flags: per WPS spec, a choice of ESS, IBSS or both.  However, IBSS is not supported by Atheros.

- config_methods: per WPS spec, a choice of user interface methods.

- wps_state: per WPS spec, configured or not configured. This may not matter for a supplicant.

- rf_bands: per WPS spec, 2.4 or 5 GHz or both. Not clear how or if this is actually used.

- Option to disable use of WPS.

- Option to disable use of UPnP with WPS (however, UPnP is not supported anyway).

- misc. descriptive information, per WPS spec. Some of this may be visible to users on e.g. an external registrar or similar.

Supplicant (station) network sets. The supplicant can be configured with any number of network configurations, and will cycle through these until it finds one that works.  Beware of unbounded accumulation of configurations... a better policy may be to allow only one (e.g. using “clean” option of the CONFIGME command).

The configuration fields are defined in the source code: apps/wpa2/wpa_supplicant/wpa_supplicant.conf

TODO: add a statement of what we'll support and what not.

Configuring an access point for open, unencrypted use.

To configure the AP to be totally open and unencrypted you need the

following values in the per-BSS configuration file:

ieee8021x=0

auth_algs=1

wpa=0

wpa_key_mgmt=

(or equivalently, the field "wpa_key_mgmt" may be commented out).

(and you need to comment out wep key info?).

Configuring an access point for simple WEP use.

To configure the AP to use WEP-PSK ("WEP-open") you need the following values in the per-BSS configuration file:

ieee8021x=0

wpa=0

wpa_key_mgmt=

(or equivalently, the field "wpa_key_mgmt" may be commented out)

auth_algs=1

wep_default_key=0

(wep_default_key can be 0, 1, 2 or 3 but 0 is most common; hostapd uses 0-based numbering of keys)

wep_key0="abcde"

(Use appropropriate wep key! Use wep_key<n> for wep_default_key=<n>).

(Wep keys must be double quoted strings of 5 or 13 chars, or hex numerals

of 10 or 26 characters; other wep key sizes are NOT supported).

Configuring an access point for WPA/TKIP/PSK use:

To configure the AP to use WPA-TKIP-PSK, set in the per-BSS configuration file:

ieee8021x=1

auth_algs=1

wpa=1

wpa_key_mgmt=WPA-PSK

wpa_pairwise=TKIP

and also set one of wpa_psk or wpa_passphrase (may need to remove wep key info?).

Configuring an access point for WPA2/CCMP/PSK use:

To configure the AP to use WPA2-CCMP-PSK, set in the per-BSS configuration file:

ieee8021x=1

auth_algs=1

wpa=2

wpa_key_mgmt=WPA-PSK

wpa_pairwise=CCMP

and also set one of wpa_psk or wpa_passphrase (may need to remove wep key info?).

Configuring an access point for mixed WPA PSK use:

To configure the AP to work with a per-station choice of WPA or WPA2 and TKIP or CCMP (aka AES) according to the capability of each station, set in the per-BSS configuration file:

ieee8021x=1

auth_algs=1

wpa=3

wpa_key_mgmt=WPA-PSK

wpa_pairwise=CCMP TKIP

and also set one of wpa_psk or wpa_passphrase (may need to remove wep key info?).

Configuring an access point for WEP 8021x EAP based authentication

To configure WEP with 8021x EAP authentication (e.g. via RADIUS server):

auth_algs=1

ieee8021x=1

eapol_version=2

wep_key_len_broadcast=13

wep_key_len_unicast=13

(or key length 5 could be used)

#wep_rekey_period=300       # this is default value

own_ip_addr=192.168.1.1     #AP's own IP address

(this obviously will need to be fixed to your local value)

auth_server_addr=192.168.1.20    #Radius servers IP address

(this also will need to be fixed for local value)

auth_server_port=1812            #Radius server listening Port

(this may need to be fixed)

auth_server_shared_secret=secret #secret password used b/w ap & server

(this will need to be fixed)

wps_disable=1

Configuring an access point for WPA 8021x EAP based authentication

To configure WPA and/or WPA2 with 8021x EAP authentication (e.g. RADIUS server):

auth_algs=1

ieee8021x=1

eapol_version=2

own_ip_addr=192.168.1.1     #AP's own IP address

(fix for local value)

auth_server_addr=192.168.1.20    #Radius servers IP address

(fix for local value)

auth_server_port=1812            #Radius server listening Port

(fix for local value)

auth_server_shared_secret=secret #secret password used b/w ap & server

(fix for local value)

# wpa:  set to 1 to enable WPA1 version,

#     set to 2 to enable WPA2 	version ,

#      set to 3 to enable both WPA1 as well as WPA2 i.e mixed key management mode

wpa=3

wpa_key_mgmt=WPA-EAP #set to WPA-EAP to enable WPA 8021x EAP authentication

wpa_pairwise=TKIP CCMP   #set to either TKIP/CCMP, Mixed support set to TKIP CCMP

#wpa_group_rekey=600 #group Rekeying interval

#rsn_preauth=1      #to be set to 1 to support pre authentication in ap's

#rsn_preauth_interfaces=eth0 #interface which accepts pre authentication

(this may need to be changed)

wps_disable=1

Configuring an access point from an external source:

The following command sets up a meta-session to configure the access point from any available external source (via WiFi or UpnP):

CONFIGME [pin=<pin>] [timeout=<seconds>]

where pin is a valid 8 decimal digit numeral (note that the last digit is a checksum of the first seven).

If no pin is given, it defaults to 00000000 which indicates push button method.

The PIN (or push button method) must be the same as used with the external source.

The default timeout is 120 seconds (as appropriate per WPS spec for push button method).

If successful, the entire hostapd process (not just that BSS) is restarted in place.

Alternately, using a command can be avoid by modifying the per-BSS configuration:

wps_configured=0

wps_default_pin=<pin>

The PIN can then be entered into the external source at any time to begin WPS operation with the given PIN.

Configuring a station to be configured via WPS:

The following per-station configuration contains no network configuration but makes the station ready to be configured via WPS.  Please refer to examples/config1.sta for more documentation:

ctrl_interface=/var/run/wpa_supplicant

update_config=1

wps_property={

wps_disable=0

wps_upnp_disable=0

version=0x10

# auth_type_flags : capabilities of network authentication

# Open    : 0x0001

# WPAPSK  : 0x0002

# Shared  : 0x0004

# WPA     : 0x0008

# WPA2    : 0x0010

# WPA2PSK : 0x0020

auth_type_flags=0x0022

# encr_type_flags : capabilities of network encryption

# None : 0x0001

# WEP  : 0x0002

# TKIP : 0x0004

# AES  : 0x0008

encr_type_flags=0x000c

# conn_type_flags : capabilities of connection

# ESS  : 0x01

# IBSS : 0x02

conn_type_flags=0x01

# config_methods : supported configuration methods

# USBA                 : 0x0001

# Ethernet             : 0x0002

# Label                : 0x0004

# Display              : 0x0008

# External NFC Token   : 0x0010

# Integrated NFC Token : 0x0020

# NFC Interface        : 0x0040

# Push Button          : 0x0080

# Keypad               : 0x0100

config_methods=0x01ca

# wps_state : Wi-Fi Protected Setup State

# Not congigurad : 0x01

# Configured     : 0x02

# Reserved       : 0x00, 0x03 - 0xFF

wps_state=0x01

# rf_bands : supported RF bands

# 2.4GHz : 0x01

# 5.0GHz : 0x02

rf_bands=0x01

# manufacturer : the name of manufacturer

# the length of manufacturer must be less than 64 bytes

manufacturer="Atheros"

# model_name : the name of model

# the length of model_name must be less than 32 bytes

model_name="WPS_SUPPLICANT_STATION"

# model_number : model number

# the length of model_number must be less than 32 bytes

model_number="01234567"

# serial_number : serial number

# the length of serial_number must be less than 32 bytes

serial_number="01234567"

# dev_category : device category (refer to examples/config1.sta)

dev_category=1

dev_sub_category=1

# dev_oui : device OUI

# the length of dev_oui must be 4 bytes (hex)

dev_oui=0050f204

# dev_name : device name

# the length of dev_name must be less than 32 bytes (hex)

dev_name="AtherosPC"

# dev_name : os_version

# for futures

os_version=0x00000001

# newsettings_command : optional sharing mechanism.

# If set (and not empty), the given command will be executed

# (in the background) whenever a WPS settings are received

# (e.g. from M8 message).

# The command is passed an argument which is a file that has

# been written containing the raw WPS settings information elements.

#

#newsettings_command="repeater_pass_configuration"

}

Configuring a station network description

wpa_supplicant can be configured with a number of network description blocks.  wpa_supplicant will cycle through the network description blocks until it finds one that works.  Since this cycling can be time-consuming, in general one should avoid multiple network descriptions unless there is a clear advantage.  See wpa_supplicant/wpa_supplicant.conf for documentation of network blocks, including a number of examples (note that this document does not include WPS support, for which see preceding section).